Traffic Light Project Overview

Microcontroller Lab 3: Buttons and Switches (STM32F103C6, Proteus)

This document summarizes the four-way traffic light we built for the STM32F103C6 and simulated in Proteus, following the Lab 3 handout on buttons, debouncing and finite state machines. Exercises 1 to 9 are done and the core of Exercise 10 is written. The video and the report are still to do.

# 1. What the lab asks for

- A crossroad with 12 LEDs (4 red, 4 amber, 4 green) and four 7-segment displays, two per road.

- Three buttons: mode, increase, and set.

- Four modes. Mode 1 is the normal cycle. Modes 2 to 4 edit the red, amber and green durations (1 to 99) while that color blinks at 2 Hz.

# 2. Design decisions

| Item | Choice |
| --- | --- |
| Road LEDs | PA0 to PA2 for road 1 (red, amber, green), PA3 to PA5 for road 2 |
| Digit enables | PA6 to PA9, through PNP transistors (active-low) |
| Segments a to g | PB8 to PB14, shared by all four displays (multiplexing) |
| Buttons | PB0 to PB2, active-low, with 10k pull-up resistors |
| Timer | TIM2, prescaler 7999, period 9, giving a 10 ms tick on the 8 MHz internal clock |
| Time rule | red = amber + green, so the two roads stay in sync |
| Defaults | red 5 s, amber 2 s, green 3 s |

# 3. Exercise by exercise

- Exercise 1 (FSM sketch): four diagrams: the button FSM (released, pressed, held more than 1 s), the mode FSM (1 to 4 and wrap), the four-phase traffic cycle, and the edit-mode behavior.

- Exercise 2 (Proteus schematic): we reviewed the schematic and listed the fixes: current-limiting resistors on the LEDs, 220 Ω resistors on the segment lines, 1k base resistors on the PNPs, and matching net labels. We also explained why four displays share one set of a to g lines.

- Exercise 3 (CubeMX project): pin modes and initial levels (digit enables and segments start High), TIM2 at 10 ms with its interrupt, HAL_TIM_Base_Start_IT, and a pin_map.h header with pin names because no user labels were set. We also explained why all displays are lit before any firmware runs.

- Exercise 4 (timer independence): a software_timer module where durations are written in milliseconds and converted to ticks using TIMER_CYCLE. Changing the TIM2 period then needs only two edits. Three runs (1 ms, 10 ms, 100 ms) prove it.

- Exercise 5 (buttons): input_reading (two-buffer debounce for three buttons with a 1 s hold flag), input_processing (one reusable button FSM with a 500 ms auto-repeat), and the first version of the mode FSM.

- Exercise 6 (displays and blinking): output_display with multiplexed scanning, the segment code table and LED helpers. The mode number is shown on the right two digits and the selected color blinks at 2 Hz.

- Exercises 7, 8 and 9 (edit red, amber, green): one shared edit routine. Button 2 changes a temporary value (wrapping from 99 to 1) and button 3 saves it with validity checks. Leaving without Set discards the change.

- Exercise 10 (integration): the Mode 1 cycle with countdown digits. We also answered why Mode 1 showed 01 and 00 (an old placeholder) and why saved values survive (static variables).

# 4. File structure

| File | Role |
| --- | --- |
| software_timer.c / .h | Millisecond timers built from the 10 ms tick |
| timer.c | Interrupt callback: runs the timers and reads buttons |
| input_reading.c / .h | Debounced button driver |
| input_processing.c / .h | Button FSM (press, hold, repeat) |
| output_display.c / .h | 7-segment scan and LED helpers |
| fsm_traffic.c / .h | Mode FSM, edit logic, normal-mode cycle |
| pin_map.h | Pin names |
| main.c | Initialization, then fsm_run() and the display scan in the loop |

# 5. Problems fixed along the way

- The handout’s Program 1.1 is buggy, so we used the two-buffer version from Program 1.2 instead.

- Floating pins lit every display before the program loaded.

- The “invalid storage class” build error was caused by a missing closing brace in main().

- We answered questions on the PNP wiring, how the digit COM pins connect, and the red = green + amber rule.

# 6. What remains

- Check that the 220 Ω segment resistors and the 330 Ω LED resistors are in the schematic. The last screenshot showed 1k LED resistors and no visible segment resistors.

- Run the full Mode 1 test table and the mode and edit tests from the earlier steps.

- Record the video, showing every feature in the specification: the normal cycle, the mode cycle, each edit mode with blinking, wrap at 99, a saved value taking effect, and a discarded edit.

- Write the report with one section per exercise: the diagrams, schematic, CubeMX screenshots, key code with explanations, and test results. State the assumptions (the time rule, rejected values, Mode 1 restarting at phase 0).

- Submit the report, the code and the video on BKeL.